Crypto-RFID Subsystem — RTL Hierarchy

Hierarchy and File Structure

# 1. Top-Level Design

The top-level RTL module is:

crypto_rfid_top

It acts as the APB slave wrapper and integrates the Digital Baseband (DBB) and PRESENT-80 cryptographic subsystem.

The top-level responsibilities include:

APB address decoding and register interface

CTRL/STATUS register handling

Key and data register storage

APB read/write handshake and wait-state generation

Mode selection between Live, DBB Loopback, and Crypto Loopback

DBB-to-PRESENT data handoff

PRESENT-80 control and result handling

DATA_OUT register handling

# 2. Overall RTL Hierarchy

crypto_rfid_top
                                |
              +-----------------+------------------+
              |                                    |
              ▼                                    ▼
           dbb_top                            present_core
              |                                    |
      +-------+-------+                    PRESENT-80 internals
      |       |       |
      ▼       ▼       ▼
   DBB     DBB     DBB
 modules  modules  modules

# 3. Top-Level Module: crypto_rfid_top

crypto_rfid_top

Main interfaces:

APB interface:
psel, penable, pwrite, paddr, pwdata, prdata, pready

RF interface:
rfid_rx_bit, rfid_tx_bit

Main internal blocks:

crypto_rfid_top
│
├── APB interface / transfer detection
├── Register file
│   ├── CTRL_REG
│   ├── KEY_0
│   ├── KEY_1
│   ├── KEY_2
│   ├── DATA_0
│   └── DATA_1
├── Mode/control logic
├── DBB replay-busy / timeout logic
├── DATA_OUT register
├── STATUS logic
├── dbb_top
└── present_core

# 4. DBB Hierarchy

The top-level wrapper instantiates dbb_top. The DBB-related RTL files compiled for the design are:

dbb_top.v
├── dbb_sync.v
├── dbb_oversampler.v
├── dbb_manchester_decoder.v
├── dbb_majority_voter.v
├── dbb_byte_assembler.v
├── dbb_byte_framer.v
├── dbb_frame_ctrl_fsm.v
└── dbb_loopback_serializer.v

These modules together implement the digital baseband processing.

At the wrapper interface, dbb_top provides the Live Mode data handoff through cipher_data_in and cipher_data_valid. In DBB Loopback Mode, the wrapper provides DATA_0/DATA_1 and lb_load, and receives frame_data_out0/frame_data_out1 and data_ready.

The DBB also provides frame_parity_error and accepts data_read_ack from the wrapper.

# 5. PRESENT-80 Hierarchy

The wrapper instantiates the PRESENT-80 core. The PRESENT-related RTL files used in the simulation are:

present_core.v
present_key_scheduling.v
present_p_layer.v
present_round.v
present_sbox.v
present_s_layer.v

crypto_rfid_top
       |
       ▼
 present_core
       |
       +── present_key_scheduling
       +── present_round
       +── present_sbox
       +── present_s_layer
       +── present_p_layer

The exact instantiation relationship between these PRESENT modules is determined by the contents of present_core.v and the corresponding RTL files.

# 6. Functional Data Paths

## Mode 0 — Live Mode

RF input
  │
  ▼
rfid_rx_bit
  │
  ▼
dbb_top
  │
  │ complete 64-bit data + valid
  ▼
cipher_data_in
  │
  ▼
present_core
  │
  ▼
cipher_data_out
  │
  ▼
DATA_OUT registers

PRESENT-80 is therefore used in the Live Mode path.

## Mode 1 — DBB Loopback

CPU
 │
 ├── DATA_0
 └── DATA_1
       │
       ▼
   crypto_rfid_top
       │
       │ lb_load
       ▼
    dbb_top
       │
       ▼
frame_data_out0/1
       │
       ▼
 data_out_reg
       │
       ▼
 CPU reads DATA_OUT_0/1

PRESENT-80 is not involved in Mode 1.

## Mode 2 — Crypto Loopback

CPU
 │
 ├── DATA_0
 └── DATA_1
       │
       ▼
crypto_rfid_top
       │
       ▼
cipher_data_in
       │
       ▼
present_core
       │
       ▼
cipher_data_out
       │
       ▼
data_out_reg
       │
       ▼
CPU reads DATA_OUT_0/1

Mode 2 directly feeds the CPU-provided 64-bit plaintext to PRESENT-80.

# 7. APB Control Hierarchy

The APB master communicates only with crypto_rfid_top. The wrapper then controls the internal DBB and PRESENT blocks.

APB Master / CPU
                     │
                     ▼
             crypto_rfid_top
                     │
          +----------+----------+
          │                     │
          ▼                     ▼
       dbb_top             present_core

The wrapper therefore acts as the integration layer between the APB interface, DBB, and cryptographic core.

# 8. Simulation/Testbench Hierarchy

The simulation testbench is:

tb / rfid_tb_top.sv
        │
        ▼
rfid_top.v
        │
        ▼
crypto_rfid_top
        │
        ├── dbb_top
        │
        └── present_core

The testbench drives the clock, reset, APB signals, and rfid_rx_bit, and checks the APB outputs and functional results.

# 9. Xcelium Compilation File Set

The final simulation is compiled using the following RTL files.

## DBB

dbb_byte_assembler.v
dbb_byte_framer.v
dbb_frame_ctrl_fsm.v
dbb_loopback_serializer.v
dbb_majority_voter.v
dbb_manchester_decoder.v
dbb_oversampler.v
dbb_sync.v
dbb_top.v

## PRESENT-80

present_core.v
present_key_scheduling.v
present_p_layer.v
present_round.v
present_sbox.v
present_s_layer.v

## Top-level and testbench

rfid_top.v
rfid_tb_top.sv

# 10. Final Integration Hierarchy

rfid_tb_top.sv
                          │
                          ▼
                       rfid_top
                          │
                          ▼
                  crypto_rfid_top
                   /             \

▼               ▼
              dbb_top       present_core
                │                 │
                │                 ├── key scheduling
                │                 ├── rounds
                │                 ├── S-box
                │                 ├── S-layer
                │                 └── P-layer
                │
                ├── synchronizer
                ├── oversampler
                ├── Manchester decoder
                ├── majority voter
                ├── byte assembler
                ├── byte framer
                ├── frame-control FSM
                └── loopback serializer

Note: The exact internal hierarchy below dbb_top and present_core depends on the module instantiations in their respective RTL files.